Kalman Filter

📌 서술 구조 (수정본)

1. 배경 및 문제 정의

자율주행 차량의 위치는 주로 SLAM(Simultaneous Localization and Mapping)을 통해 추정되지만, 실제 주행 실험에서는 동일한 위치에서도 소수점 두세 자리 수준의 미세한 좌표 변동이 지속적으로 발생 하였다. 이러한 미세한 오차는 협소하고 곡률이 큰 트랙 환경에서는 누적되어 차량의 경로 추종 정확도와 안정성에 직접적인 영향을 미칠 수 있다.

이에 따라 본 연구에서는 SLAM 기반 위치 추정값을 보정하기 위해 Kalman Filter(KF)를 적용하였으며, 센서 데이터의 잡음 특성을 반영하고 불확실성을 줄임으로써 보다 안정적이고 신뢰성 높은 위치 추정을 목표로 하였다.

1.Background and Problem Definition

The vehicle’s position was estimated using SLAM (Simultaneous Localization and Mapping); however, during real-world experiments, slight coordinate fluctuations were observed even when the vehicle remained at the same location—typically from the second or third decimal place.

Although seemingly minor, such positional drift can accumulate in a narrow and highly curved track environment, significantly affecting trajectory-tracking accuracy and overall driving stability.

To mitigate this issue, a Kalman Filter (KF) was applied to refine the SLAM-based position estimates. By incorporating the sensor noise characteristics and reducing estimation uncertainty, the proposed approach aimed to achieve more stable and reliable localization for autonomous driving.

2. 방법론

- 센서 구성: LiDAR와 IMU 데이터를 이용하여 SLAM 기반 위치 추정을 수행하였으며, 환경적 제약으로 인해 GPS는 사용하지 않았다.

- 상태 추정 보정: SLAM의 출력값을 Kalman Filter(KF)로 후처리하여, 단기적인 위치 흔들림을 억제하고 더 신뢰성 있는 위치 추정값을 확보하였다.

- 로그 데이터 분석 및 파라미터 조정: SLAM 실험 로그는 Anaconda 환경의 Python(Pandas)을 활용하여 전처리하였다.

  - 중복된 타임스탬프를 제거하고 시간 순으로 정렬하였다.

  - 좌표값 (x,y)의 평균 및 분산을 계산하여 초기 센서 잡음 공분산 R을 추정하였다.

  - 이후 반복적인 주행 테스트를 통해 노이즈 파라미터 (Q,R)를 실험적으로 조정하였다.

    - 위치 추정값의 변동이 과도하게 클 경우, 센서 측정값을 더 신뢰하도록 R을 감소시켰다.

    - 위치 추정값의 반응이 느리거나 지연되는 경우, 모델 예측을 더 반영하도록 Q를 증가시켰다.

  - 이러한 적응적 튜닝 과정을 통해 SLAM의 출력이 점차 안정화되었으며, 동일 위치에서 반복 측정 시 일관된 추정값을 유지할 수 있었다.

  - 파라미터가 안정화된 이후, 조정 과정에서 사용된 ROS 퍼블리셔 및 관측용 로그 데이터는 모두 삭제하여 후속 실험에 영향을 주지 않도록 관리 하였다.

- 이 코드는 SLAM 로그 데이터를 전처리하고, 센서 잡음 특성을 분석하여 Kalman Filter의 초기 공분산 행렬 R을 추정하는 데 사용되었다.

- 파라미터 조정 완료 후에는 모든 임시 ROS 퍼블리셔 및 관측 로그를 삭제하여, 실험 데이터의 일관성과 신뢰성을 유지하였다.

English Version (영문 버전)

Methodology

- Sensor Configuration: SLAM-based localization was performed using LiDAR and IMU data, while GPS was excluded due to environmental constraints.

- State Estimation Refinement: The SLAM outputs were post-processed with a Kalman Filter to suppress short-term fluctuations and achieve more reliable position estimates.

- Log Data Analysis and Parameter Adjustment: The SLAM experiment logs were preprocessed using Python (Pandas) in the Anaconda environment.

  - Duplicate and overlapping entries were removed and sorted.

  - The mean and variance of position coordinates (x,y) were computed to estimate the initial sensor noise covariance R.

  - During iterative tests, the noise parameters (Q,R) were empirically tuned according to the following principles:

    - If the position estimate fluctuated excessively, R was decreased to rely more on sensor measurements.

    - If the estimate lagged or responded slowly, Q was increased to rely more on the model prediction.

  - This adaptive tuning process gradually stabilized SLAM outputs, maintaining consistent position values under repeated real-vehicle experiments.

  - After parameter stabilization, the temporary ROS topics and published observation logs used for tuning were deleted, ensuring clean data management for subsequent experiments.

CODE

import pandas as pd

import numpy as np

# Load SLAM log data

df = pd.read_csv("slam_log.csv")

# Remove duplicate timestamps and sort

df = df.drop_duplicates(subset='timestamp').sort_values('timestamp')

# Calculate mean and variance for position coordinates

x_mean, y_mean = df['x'].mean(), df['y'].mean()

x_var, y_var = df['x'].var(), df['y'].var()

print(f"Mean Position: ({x_mean:.4f}, {y_mean:.4f})")

print(f"Variance (x, y): ({x_var:.6f}, {y_var:.6f})")

# Initial R matrix (measurement noise covariance)

R = np.diag([x_var, y_var])

print("Initial Measurement Noise Covariance R:\n", R)

This script was used to preprocess SLAM logs, calculate noise characteristics, and determine the initial covariance R for the Kalman Filter.

After parameter stabilization, all temporary ROS publishers and observation logs were deleted to maintain clean experiment data management.

3. 결과 및 분석 (Results and Analysis)

- 협소하고 곡률이 큰 트랙 환경에서도 Kalman Filter 기반 SLAM 보정 시스템을 적용하여, 일부 구간에서 위치 추정의 미세한 흔들림이 완화되는 효과를 확인할 수 있었다.

- 그러나 위치 추정값이 완전히 일정하게 수렴하지는 않았으며, 구간에 따라 갑작스러운 변화나 불안정한 추정이 여전히 발생하였다. 이는 Kalman Filter의 동역학 모델이 실제 차량의 운동 특성을 충분히 반영하지 못했고, 모델 불확실성(Q)과 측정 잡음(R)의 수치를 정확히 구현하기 어려웠던 점이 주요 원인으로 분석된다.

- 따라서 보정 효과는 전반적으로 제한적이었으며, 일부 구간에서는 오히려 필터가 불안정한 예측값에 과도하게 반응하는 현상도 관찰되었다.

- 그럼에도 불구하고 본 실험을 통해 센서 기반 위치 추정의 불확실성이 실제 주행 안정성에 미치는 영향, 그리고 모델링 정밀도가 Kalman Filter 성능에 미치는 민감도를 직접 검증할 수 있었다. 이는 향후 상위 연구 단계에서 정확한 동역학 모델 정의, Adaptive Kalman Filter 구조, 비선형 확장(EKF, UKF) 적용 등의 방향으로 발전시킬 수 있는 기반이 된다.

- 모델 불확실성과 동역학 복잡성으로 인해 안정적인 보정 성능을 확보하기 어려워, 본 시스템에서는 실험 단계까지만 적용하였다.

3. Results and Analysis

- In a narrow and high-curvature track environment, the application of a Kalman Filter–based SLAM correction system showed partial improvement in localization stability, reducing minor oscillations in certain segments.

- However, the estimated positions did not fully converge to a stable value, and sudden variations or inconsistencies were still observed in some sections. This behavior was mainly attributed to the limited accuracy of the vehicle’s dynamic model used in the Kalman Filter and the difficulty in accurately defining the process (Q) and measurement (R) noise covariances.

- Consequently, the overall correction effect was modest, and in some cases, the filter exhibited overreaction to unstable prediction states, leading to temporary estimation deviations.

- Nevertheless, this experiment provided valuable insight into how sensor-based localization uncertainty affects driving stability and how the precision of the dynamic model directly influences Kalman Filter performance. These findings suggest that future research can enhance the results by incorporating a more accurate dynamic model, Adaptive Kalman Filter structures, and nonlinear extensions such as EKF or UKF to achieve higher stability and robustness.

- Due to model uncertainty and dynamic complexity, stable correction performance was difficult to maintain, so it was applied only at the experimental stage.

4. 한계 및 향후 개선 (Limitations and Future Improvements)

- 학부 수준에서는 센서 잡음 모델(Q,R)을 수학적으로 완전하게 정의하기 어려워, 테스트 기반의 실험적 보정에 의존할 수밖에 없었다. 특히, 불확실성 공분산 행렬을 정량적으로 모델링하기 위한 데이터가 제한되어 있어, 모델 신뢰도와 필터의 반응성 간의 균형 조정이 어려웠다.

- 또한 동역학 모델의 세부 표현 한계로 인해, Kalman Filter가 예측 단계에서 실제 차량의 운동 특성을 충분히 반영하지 못하는 경우가 발생하였다. 이로 인해 필터의 보정 효과가 일부 구간에서 미미하거나 불안정한 양상을 보였다.

- 향후 연구에서는 Adaptive Kalman Filter를 적용하여 주행 중 센서 노이즈 특성의 변화를 실시간으로 반영하고, 센서 타임스탬프 정밀 동기화 및 고주기 데이터 수집 환경을 구축함으로써 보다 정교한 추정이 가능할 것으로 기대된다.

- 나아가 석·박사 수준 연구에서는 잡음 모델링의 수학적 정식화, 필터 안정성 및 수렴성 증명, 그리고 **비선형 확장(EKF, UKF 등)**을 포함한 이론적 확장을 통해 보다 높은 수준의 신뢰성과 일반화를 달성할 수 있을 것이다.

4.Limitations and Future Improvements

- At the undergraduate level, it was difficult to mathematically define the sensor noise covariance matrices (Q and R) precisely, making the system rely primarily on empirical, test-based calibration. Due to limited data for quantitatively modeling uncertainty, balancing between model reliability and filter responsiveness remained a challenge.

- Moreover, the limited fidelity of the vehicle’s dynamic model prevented the Kalman Filter from fully capturing real vehicle behavior during the prediction step. As a result, the correction performance was sometimes marginal or unstable in certain conditions.

- Future research could enhance estimation precision by implementing an Adaptive Kalman Filter that dynamically updates noise parameters in real time and by improving sensor timestamp synchronization under high-frequency data sampling.

- At the graduate and doctoral research level, rigorous noise modeling, stability and convergence proofs, and nonlinear extensions such as EKF and UKF could further strengthen the theoretical foundation and achieve higher reliability and generalization performance.

🔹 Kalman Filter Design Overview

1️⃣ Concept

The Kalman Filter is applied to refine the vehicle’s position estimated by SLAM, by combining the system model from MATLAB Simulink with measurement data from ROS sensors. It operates in two main stages — Prediction and Update — as shown below.

Here,

- x: state vector [x,y,heading,v]

- F: state transition matrix (from Simulink vehicle dynamics)

- u: control input (acceleration, steering)

- P: covariance matrix (uncertainty)

- Q,R: process and measurement noise

- H: measurement matrix (maps SLAM outputs to state space

The filter runs in real-time within a ROS2 node implemented in Python,

integrating SLAM measurements and Simulink-published vehicle dynamics.

| 역할 | 담당 시스템 | 설명 |
| --- | --- | --- |
| SLAM | ROS | LiDAR + IMU → 위치(x, y, heading) 추정 |
| Vehicle Dynamics (F, u) | MATLAB Simulink | 차량 동역학 계산 및 ROS로 퍼블리시 |
| Kalman Filter (보정) | Python ROS2 Node | SLAM 위치값을 Simulink 모델 기반으로 보정 |
| Output | ROS Topic /kf_output | 보정된 위치를 MATLAB / RViz / 제어 모듈에 전달 |

| Role | Platform / System | Description |
| --- | --- | --- |
| SLAM | ROS | Estimates vehicle position (x, y, heading) using LiDAR and IMU data |
| Vehicle Dynamics (F, u) | MATLAB Simulink | Computes dynamic state transition and publishes to ROS |
| Kalman Filter (Correction) | Python ROS2 Node | Refines SLAM positions based on Simulink vehicle model |
| Output | ROS Topic /kf_output | Publishes corrected position to MATLAB, RViz, or control modules |

Summary

- SLAM의 미세한 위치 흔들림(소수점 2–3자리 수준)을 Simulink 기반 모델로 보정.

- MATLAB에서 계산된 차량 동역학 모델(F, u)을 ROS2로 전달받아 실시간 보정 수행.

- Python 노드는 오차 공분산을 이용해 SLAM의 불확실성을 줄이고 안정적인 위치 추정 확보.

- 결과적으로 협소한 트랙 환경에서도 보다 부드럽고 안정적인 주행을 구현.

Summary

- The Kalman Filter was applied to mitigate small oscillations in SLAM-estimated positions (at the 2nd–3rd decimal level).

- MATLAB Simulink provided the vehicle dynamic model (F, u), which was transmitted to ROS2 in real time.

- The Python ROS2 node fused these inputs to reduce covariance-based uncertainty and stabilize localization.

- As a result, the vehicle maintained smoother and more stable motion even in narrow or curved tracks.

CODE

#!/usr/bin/env python3

import rclpy

from rclpy.node import Node

import numpy as np

from geometry_msgs.msg import Point

from std_msgs.msg import Float64MultiArray

class KalmanFilterNode(Node):

def __init__(self):

super().__init__('kalman_filter_node')

# Subscribers

self.create_subscription(Point, 'location', self.cb_location, 10)      # (x, y, heading)

self.create_subscription(Float64MultiArray, 'F_matrix', self.cb_F, 10) # Simulink에서 전달되는 동역학 모델

self.create_subscription(Float64MultiArray, 'u_vector', self.cb_u, 10) # Simulink에서 전달되는 제어 입력

# Publisher (보정된 상태 퍼블리시)

self.kf_pub = self.create_publisher(Point, 'kf_output', 10)

# ==============================

#  Kalman Filter 초기 설정

# ==============================

self.x = np.zeros((4, 1))           # [x, y, heading, v]

self.P = np.eye(4) * 0.1            # 초기 공분산 (불확실성)(수정필요)

self.Q = np.diag([1e-4, 1e-4, 1e-5, 1e-5])  # 프로세스 노이즈

self.R = np.diag([1e-3, 1e-3])              # 측정 노이즈

# 최신 데이터 저장용 변수

self.slam_x = None

self.slam_y = None

self.slam_heading = None

self.F = np.eye(4)

self.u = np.zeros((1, 1))

self.get_logger().info("✅ Kalman Filter Node initialized successfully.")

# =========================================================

# SLAM 데이터 수신 콜백 (x, y, heading)

# =========================================================

def cb_location(self, msg: Point):

self.slam_x = msg.x

self.slam_y = msg.y

self.slam_heading = msg.z

# 상태 갱신

self.x[0, 0] = self.slam_x

self.x[1, 0] = self.slam_y

self.x[2, 0] = self.slam_heading

# 데이터가 모두 존재하면 필터 실행

if hasattr(self, 'F') and hasattr(self, 'u'):

self.run_kalman_filter()

def cb_F(self, msg):

self.F = np.array(msg.data).reshape(4,4)

def cb_u(self, msg):

self.u = np.array(msg.data).reshape(1,1)

# =========================================================

# 칼만 필터 메인 루프

# =========================================================

def run_kalman_filter(self):

# -------------------------------

# 예측 단계 (Prediction)

# -------------------------------

x_pred = self.F @ self.x + self.u

P_pred = self.F @ self.P @ self.F.T + self.Q

# -------------------------------

# 측정 단계 (Update)

# -------------------------------

z = np.array([[self.slam_x], [self.slam_y]])

H = np.array([[1, 0, 0, 0],

[0, 1, 0, 0]])

K = P_pred @ H.T @ np.linalg.inv(H @ P_pred @ H.T + self.R)

self.x = x_pred + K @ (z - H @ x_pred)

self.P = (np.eye(4) - K @ H) @ P_pred

# -------------------------------

# 보정 결과 Publish

# -------------------------------

corrected = Point()

corrected.x = float(self.x[0, 0])

corrected.y = float(self.x[1, 0])

corrected.z = float(self.x[2, 0])  # heading 유지

self.kf_pub.publish(corrected)

self.get_logger().info(

f"[KF] x={corrected.x:.3f}, y={corrected.y:.3f}, v={self.x[3,0]:.3f}"

)

# =========================================================

# 메인 함수

# =========================================================

def main(args=None):

rclpy.init(args=args)

node = KalmanFilterNode()

rclpy.spin(node)

node.destroy_node()

rclpy.shutdown()

if __name__ == '__main__':

main()